Assignment 3 if for while conditions structure

Released Date: Oct 15th

Version 1.0

Format: Name the document in the combination of name、ID and No. of Assignment.

Example: Ping Yi_23_Assignment1.doc

Email the document to “pingy@wxit.edu.cn” before Oct 18th.

Task 1 4 LEDs are connected to Arduino Uno, try to use for cycle structure to make a 1s shift effect.

任务1 使用4个LED连接至Arduino Uno，尝试使用for循环结构，实现1s位移点亮的效果。

You could use Tinker CAD or Proteus for the verifying.可使用在线工具Tinker CAD或Proteus进行仿真。

Fig 1 LED Shift Light

| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 | int i=1; void setup() { for(i=1;i<5;i++){ pinMode(i,OUTPUT); } } void loop() { for(i=1;i<5;i++){ digitalWrite(i,HIGH); delay(1000); } for(i=1;i<5;i++){ digitalWrite(i,LOW); delay(1000); } } |
| --- | --- |

Task 2 One LED and a POT are connected to Arduino, define a function named getVoltage to calculate the real time voltage of the POT, if the value is larger than 2.5V, then turn on the LED.

任务2 一个LED和一个电位器与Arduino Uno相连，定义一个函数名为getVoltage，用于计算POT段子的实时电压，若电压大于2.5V，则点亮LED。

Fig 2 LED Indicator

| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 | int ledPin = 3; int a0 = 0; float val = 0; void setup() { Serial.begin(9600); } void loop() { int sensorValue = analogRead(0); float voltage= sensorValue * (5.0 / 1023.0); Serial.println(voltage); delay(100); if(voltage>2.5){ digitalWrite(3,HIGH); } else{ digitalWrite(3,LOW); } } |
| --- | --- |