Assignment 4 LCD Interface

Released Date: Oct 31th

Version 1.0

Format: Name the document in the combination of name、ID and No. of Assignment.

Example: Ping Yi_23_Assignment1.doc

Email the document to “pingy@wxit.edu.cn” before Nov 6th.

Task 1 4 LEDs are connected to Arduino Uno, try to use LCD1602 to display the current state of LED been lighted.

任务1 使用3个LED连接至Arduino Uno，使用LCD1602 显示当前小灯点亮的状态，（1s位移）。

You could use Tinker CAD or Proteus for the verifying.可使用在线工具Tinker CAD或Proteus进行仿真。

Fig 1 LCD Display Indicator

|  | 1#include <LiquidCrystal.h> LiquidCrystal lcd(13, 12, 7, 6, 5, 4); String ledColor[3]={"Red","Green","Blue"}; void setup() { Serial.begin(9600); pinMode(8,OUTPUT); pinMode(9,OUTPUT); pinMode(10,OUTPUT); lcd.begin(16,2); } void loop() { for(int i = 8;i<11;i++){ int ledNo = i; digitalWrite(i,HIGH); lcd.print("Current"); delay(10); lcd.setCursor(3,1); lcd.print(ledColor[ledNo-8]); delay(1000); digitalWrite(i,LOW); } } |
| --- | --- |